INFORMATION ON CREATION AND QC OF ORTHO TILES

1)  Creation of Ada County orthophotos in Ada Co. (Modified State Plane) Coordinate System

- MBR’s (Minimum Bounding Rectangles) were defined at full section size boundaries.  The images approximated the PLS boundaries and there was no overlap.

- By using year 2000 DTM data along with a few updated areas, orthos were generated in full sections from Socet Set v.4.4.1 using VrOne v.2.22 compilation software at a half (0.5) foot pixel resolution in TIFF and TFW format.

- Each orthophoto created underwent a rigorous QC process including:

  - An Edge Check was completed to ensure that all images aligned properly to the adjacent images, using ArcView v.3.3 software.

  - “Cleaning” of images, using Corel Photopaint v.8 and v.11, to minimize the appearance of artifacts, such as, seamlines, dust particles, and scratches that appeared, from film, scanning, and orthophoto generation processes.

  - Color-Balancing of individual images using Adobe Photoshop v.7.0 to create a seamless border between multiple images.

2)  Creation of Ada County orthophotos in USGS (UTM – NAD83) Coordinate System

- MBR’s (Minimum Bounding Rectangles) were defined per the US National Grid System 1500m x 1500m tiles.

- Cleaned and color-balanced orthophotos from Ada County coordinate system, which were created in Socet Set, were then mosaicked together using Socet Set software, in groups of four (4) to ensure there would be complete coverage using the optimum amount of orthophoto imagery.

- These mosaicked orthophotos were then taken into Blue Marble Geographics Geographic Transformer v.5.0 and transformed into the UTM – NAD83 coordinate system.

- The transformed, mosaicked orthophotos were then brought back into Socet Set and the correct size tiles (1500m x 1500m/5000 pixels x 5000 pixels) were cut out of the mosaics at the precise coordinate indicated in the MBR file.

- Each new tile then underwent a rigorous QC process including:

  - An Edge Check was completed to ensure that all images aligned properly to the adjacent images, using ArcView v.3.3 software.

  - A Coordinate Check, by zooming in to the pixel level, to all four of the corner pixels to ensure that each corner had the correct pixel coordinate.

  - A Vector Data Overlay Check to ensure that the vector data overlayed the imagery in the correct position.

  - A Final “Cleaning” Check, where the tiles were brought back into Corel v.8 and were carefully checked for any anomalies.

- MrSID files were then created at a 20:1 compression ratio using the QC’d versions of the orthophoto tiles.