# File Descriptions

Valued customer:

With your deliverables, you have received CD (s) with the following information :

## Metadata.asc

An ASCII text format file which contains job specifications relating to your project including mapping control datum and survey information – also all applicable disclaimers relating to mapping limitations will be included.

A printout of US National Map Accuracy Standards is attached. *NOTE: Accuracy within “ground not visible” lines may vary in accordance with National Map Accuracy Standards (NMAS).

Data Files as requested – common delivery as follows ACAD R14 or 2000 .dwg format

DTM file – 3D generic lines and points file representing ground surface as interpreted from designated photography at date of photo. Used for contour generation or surface modeling and terrain analysis.

Generic Plan/Topography file – Generally a 3D file, unless requested in 2D, which contains all line work in a non-patterned line style. This file will contain all pertinent planimetric features relating to the project specifications, and a set of contours generated to the project specifications. This file will generally be used for engineering and design functions because of compactness and more “friendly” ease of use.

Patterned Plan/Topography file – Generally a 3D file, unless requested in 2D, which contains all line work in a segmented patterned line style. This file is generally used for plotting purposes and contains all of the features that the “generic” file contains.

Layer Lists – You may receive a printed hard copy file of the separate feature layers as listed in the Autocad format layer function for your file review.

### Project Specific Deliveries – Delivered only if required by contract

Orthophoto file – Generally a “tiff” format file combined with a “tfw” georeferencing world file which will display in a variety of softwares. These are ortho rectified photo-image files that will display in conjunction with a data set that is based within the same coordinate datum as the image. These images will be set to a resolution as defined in the job specification. The “tfw” file must accompany the “tif” file wherever it goes to display the image information within the proper active coordinate limits.

Mr_Sid file – Is a compressed image file that is much more compact and will display with a Mr_Sid viewer or in ARC/VIEW with a MR_SID extension activated.

Project Key Maps and Hard Copy Deliveries

Depending on project requirements and  or complexity you may receive a key map showing the project extents with sheet tiles and ground control information. You may also receive an index showing the position of photo-centers overlaying the project extents. As a requirement of certain projects, you could also receive hard copy plots of the project at required scale.

*Note to all customers: Your comments or feedback will be appreciated upon review of your product to insure satisfaction and completeness of delivery. If you have concerns or questions regarding a delivery please call us at ph(541)-343-8877 or fax(541)-343-8879.